February 18, 2000

Mike Robbins

POST Properties

2907 State St.

Dallas, TX  75204

Dear Mike:

On Wednesday January 26th, we met at the Addison Circle Phase II project for a preliminary walk-through of the Public Infrastructure.  After walking approximately half of the project it became clear that some general comments would be appropriate, rather than getting too specific at this time.

The signage and traffic control issues deal mainly with consistency.  The two-part uni-strut bases should be buried 33-34”, with the top between 2-3” above finished grade.  The post should be of sufficient length so that the bottom of the required signage is 7 feet above finished grade.  The posts should be set plumb.  The signage plan on Sheet P5 is not a dimensioned plan, and the placement of signs are shown in approximate locations.  The posts should generally be placed in front of adjacent trees wherever possible, and with adequate separation from nearby signs to maximize visibility.  Posts for the STOP signs and street name signs are to match the posts in Phase 1 on the west side of Quorum Dr.  A number of signs have not been placed yet and must be set prior to acceptance by the Town.  No signs shall be mounted on street or sidewalk light poles.  Any damaged signs shall be replaced.  Traffic buttons are to be checked for missing reflectors and proper adhesive, and replace all that do not meet specifications.  The buttons on Spectrum Dr. do not appear to be fastened with a properly mixed epoxy.  Replace all missing or broken buttons.

Rout and seal all unsealed street joints (Morris Ave., Spectrum Dr. north of Addison Circle, Mildred St.).  Rout and seal random cracking in pavement. Repair broken curbs.   Float epoxy in gouges in pavement.  Remove all concrete spillage from finished pavement.  Install Barrier Free Ramp at NE corner of Lewis and McEntire.  Raise curb to full height between BFRs on all four corners of Spectrum and Addison Circle.  Check and repair all “birdbaths” as needed.

Page 2									Mike Robbins

02/18/00								Post Properties

Repair all water meter vaults to remove trip hazards, and restore positive latching of doors, as needed.  For additional water/wastewater issues, see attached Utility notes.

All storm drain inlets should be checked to verify that they are cleaned out and properly grouted.  “Y” inlet aprons are to be repaired as needed at Dallas Parkway and Addison Circle, and at Spectrum and Morris.  Finish grading is required in the area between the DART easement and Lewis Place and the Hike & Bike trail, from Dallas Parkway to Quorum Dr., as is the removal of any boulders or construction debris from this area, and other areas adjacent to the project.

Of immediate concern prior to acceptance of the public infrastructure are the street signs and markings, the major work of rebuilding the curbs at the Spectrum intersection, and the Barrier Free Ramp on Lewis Pl..

Please attend to these matters so that a more detailed inspection can be made, a more manageable list generated, and the project completed and accepted.  Call Mike Murphy or myself at 972-450-2871 if you have any questions or concerns.  Thank you.

Respectfully,

David E. Wilde, Public Works Inspector

Town of Addison

cc:	Lynn Chandler, Building Official

Michael Murphy, Interim Director, Public Works

Robin Jones, Street Superintendent

attachment

Post Properties Phase II

# Lewis Place:

Build ADA ramp in radius on the Northeast corner

Mildred Place:

Replace broken concrete at alley entrance.  Remove traffic signs from light pole at Addison Circle Dr., and place on proper post in front of  tree well.  Rout and seal all pavement joints.  Replace broken pavers and clean concrete “spillage”.  Repair trip hazard at BFR at east side.  Sidewalk between street pavement and treewells is sinking.  Repair sunken brick bands that are holding water.

Morris Ave.:

Raise electric hand hole box at alley entrance.  Set sanitary sewer cleanouts to grade and place proper caps.  Paver soldier course doesn’t match throughout.  Rout and seal random cracks in pavement.  Replace and reset water meter boxes to specification at the town homes.  Finish joint sealing the curb/paver joint.  Place caps on the storm sewer cleanout pipes.  Rout and seal all street joints.  Re-establish concrete edge at Mildred Place.  Repair “birdbath” at east end of parking bay.  Clean inlet throat at east end of street.  Raise valve at the hydrant at northeast end.  Replace water meter box and lid at northeast end.  Repair and set pavers at areas where light bases were removed.  Repair area inlet apron, clean out inlet, grade and re-vegetate surrounding area.

Spectrum Dr.:

Rout and seal all pavement joints.  Replace all street buttons.  Repair redwood joint at north end.  Set pavers to match irrigation meter box on west side of street.  Remove and replace curb radii at all four corners at Addison Circle Dr. per plan.  Re-locate guardrail at south end, adjust height, and place “end of street” markings per HZ revision.

Addison Circle Dr.:

Repair broken curb in front of 5009 Addison Circle Dr.  Replace chipped traffic buttons at entrance to Esplanade.  Repair chipped curb at Barrier Free Ramp.  Place missing signage.  Redwood joints at the circle on Quorum must be sealed and repaired as needed.  Raise pavers at first treewell on northeast corner, remove concrete “spillage”.  Repair “birdbath” at northeast corner BFR at Quorum.  Repair curb/paver joint at east side of Mildred Pl.  Repair vehicular pavers at BFR.  Joint sealer is too “white” at west side of Mildred Pl.

Quorum Dr.

Seal sidewalk/curb joint north from the Circle.  Clean mortar from pavers.  Replace missing paver at Morris Ave.