DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN: Wade Geistweidt					REPORT DATE:

WEATHER: Clear & Cool with a strong North Wind			REPOR	T NO.:

TIME ARRIVED: 8:15 AM 						TEST DATE:  11-5-10

TIME DEPARTED:  5:30 PM

PROGRESS REPORT:

Wade Geistweidt with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crew is foreman by Derek Dodson and Arturo Gonzalez.  This crew consists of 6 people, 4 pieces of equipment, and numerous hand and power tools.  Austin’s crew finished wrecking the wall forms for the South Wing wall of Abutment # 1 today.  This crew also wrecked the forms for the stem wall between the North and South Wing walls.  This wall was then covered with insulated bridge blankets.  These blankets were soaked in water to help the concrete keep its heat over the weekend.  Austin also attempted to continue backfill behind bent # 2 of the Bella Lane Bridge.  This backfill was not accepted by Kleinfelder due to large rocks behind the wall.  Kleinfelder also took two field densities at this location.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT