DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN: Wade Geistweidt					REPORT DATE:

WEATHER: Cloudy with chance for rain					REPOR	T NO.:

TIME ARRIVED:  8:15 AM						TEST DATE:  4-20-11

TIME DEPARTED:  5:30 PM

PROGRESS REPORT:

Wade Geistweidt and David Hernandez with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crews are foreman by Arturo Gonzales, George, and Derek Dodson. These crews consist of 10 people, 7 pieces of equipment, and numerous hand and power tools.  Austin first and second crews worked together today on the Bella Lane Bridge.  This work included placement of 24 cubic yards on the sidewalk for the South bound lanes.  These crews also continued working on the approach slab for abutment # 4 of the Bella Lane Bridge. Erwin Steel Works had two welders onsite to continue fitting and welding the bottom slide plates of the arches at the abutment embed.  Kleinfelder cast one set of 5 4X8 cylinders for the sidewalk placement.  David Hernandez with Kleinfelder was able to perform baseline dry film thickness testing today on the Ponte Ave Bridge.  These thickness readings are required so when the intermediate and topping coats are applied a total dry film thickness can be achieved.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT