DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN: Wade Geistweidt					REPORT DATE:

WEATHER: Cloudy with chance for rain					REPOR	T NO.:

TIME ARRIVED:  8:30 AM						TEST DATE:  4-30-11

TIME DEPARTED:  11:10 AM

PROGRESS REPORT:

Wade Geistweidt with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crew is foreman by Arturo Gonzales, and Derek Dodson. This crew consists of 10 people, 7 pieces of equipment, and numerous hand and power tools.  Austin first continued working on the SW sidewalk at abutment # 2 of the Ponte Bridge.  Austin place 20 cubic yards of concrete for this section of sidewalk cap on the approach slab to the Ponte Bridge.  Austin also had a welder from Erwin Steel Works onsite to continue leveling up the low weld seams.  Austin also had a crew member grinding the welds flush after the welder was done.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT