DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN: Wade Geistweidt					REPORT DATE:

WEATHER: Clear							REPOR	T NO.:

TIME ARRIVED:  8:15 AM						TEST DATE:  5-3-11

TIME DEPARTED:  5:15 PM

PROGRESS REPORT:

Wade Geistweidt with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crews are foreman by Arturo Gonzales, George, and Derek Dodson. These crews consist of 10 people, 7 pieces of equipment, and numerous hand and power tools.  Austin first and second crews worked together today on the Bella Lane and Ponte Ave Bridges.  This work included placing 20 cubic yards for the N, S, E, & W sidewalks caps on the approach slabs to the Bella Lane Bridge.  Austin had a welder from Erwin Steel Works onsite today to continue level up of the weld seams at the Ponte Bridge arches.  Austin continued grinding these welds smooth so they could be repainted. Kleinfelder also cast one set of 5 4X8 cylinders today.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT