DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN: Wade Geistweidt					REPORT DATE:

WEATHER: Clear							REPOR	T NO.:

TIME ARRIVED:  8:15 AM						TEST DATE:  5-5-11

TIME DEPARTED:  5:35 PM

PROGRESS REPORT:

Wade Geistweidt with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crews are foreman by Arturo Gonzales, George, and Derek Dodson. These crews consist of 10 people, 7 pieces of equipment, and numerous hand and power tools.  Austin first and second crews worked together today on the Bella Lane and Ponte Ave Bridges.  This work included continuing to grind the welding level up smooth on the arches at the Ponte Bridge.  Austin crew also continued lay out of the diamond pattern on the sidewalks on both the bridges.  Austin painting subcontractor was onsite to perform sandblasting on the sidewalk at the Bella Lane Bridge.  Austin had a welder from Erwin Steel Works onsite today to continue level up of the weld seams at the Bella Lane Bridge arches.  This welder also removed the lift eyes by arch gouging the welds.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT