DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN: Wade Geistweidt					REPORT DATE:

WEATHER: Cloudy with strong South wind				REPOR	T NO.:

TIME ARRIVED:  8:15 AM						TEST DATE:  6-2-11

TIME DEPARTED:  5:00 PM

PROGRESS REPORT:

Wade Geistweidt with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crews are foreman by George, and Derek Dodson. These crews consist of 5 people, 3 pieces of equipment, and numerous hand and power tools.  Austin’s crew was tasked with reapplying the Carboguard 501 filler product on the weld seams for both the Ponte and Bella Lane Bridges.  This was done to level out the remaining low areas around the weld seams.  Kleinfelder performed the nuclear field densities behind the retaining wall today.  These test met the requirements of the project.  Austin plans to have a painting crew onsite from Scott Derr painting tomorrow June 3.  This crew will need to power wash and clean the entire surface of the Bella Lane arches before any painting can be performed.  Kleinfelder will perform a tape test to establish the level of cleaning before paint is applied.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT