DAILY REPORT SUMMARY

TO:	Town of Addison						PROJECT:  Vitruvian Park - Addison, TX

16801 Westgrove Drive								Infrastructure Phase 1D

Addison, TX  75001-5190					PROJECT NO.: 99526 – 5

TECHNICIAN:	Wade Geistweidt						 REPORT DATE:

WEATHER:  	Partly Cloudy with strong South wind and chance for rain		REPOR	T NO.:

TIME ARRIVED:  8:15 AM							TEST DATE:  9-24-10

TIME DEPARTED:  5:15 PM

PROGRESS REPORT:

Wade Geistweidt with Kleinfelder continued observation of the Vitruvian Park Phase 1D project for the Town of Addison.  Austin Bridge and Road crews are foreman by Derek Dodson, Arturo Gonzalez, and Santos.  These crews consist of 15 people, 3 pieces of equipment and numerous hand and power tools.  The first crew placed 30 cubic yards of concrete for the South Support Wall on Abutment # 2 of the Ponte Bridge.  This crew set forms and reinforcing steel until 3PM and then spent the remainder of the day pouring concrete. The second crew began backfill efforts on the Abutment # 1 footers.  Halff and Associates completed the reinforcing steel inspections for the South support wall on Abutment # 2 of the Ponte Ave Bridge. Kleinfelder cast one set of 5 4X8 cylinders for the concrete placement today on Ponte.  Kleinfelder also performed nuclear field densities on the backfill as each lift was placed.

COPIES TO:	Clay Barnett							KLEINFELDER

DRAFT