GENERAL NOTES - EROSION CONTROL

- A Storm Water Pollution Prevention Plan shall be required for this project and shall be prepared by the Owner and/or Contractor.

- Contractor shall be responsible for erosion and sediment control including control of construction access and street cleaning if necessary.  Place erosion control devices as indicated hereon prior to start of construction.  Contractor to maintain erosion control devices until time for permanent paving and landscaping.  Prior to placement of proposed paving, construction access shall be from a single location to be specified by the Contractor and the curb shall be protected in accordance with City requirements.  Vehicles shall be properly cleaned prior to entering public streets.  The Contractor shall immediately remove any construction materials tracked onto public streets.

- Inspections must be conducted at least once every fourteen (14) days and within twenty-four (24) hours of the end of a storm event of 0.5 inches or greater. As an alternative to the said inspection schedule, inspections will occur at least once every seven (7) calendar days. If this alternate schedule is used, the inspection must occur on a specific day of the week, regardless of whether or not there has been a rainfall event since the previous inspection.

- The disturbed area consists of approximately 3.5 acres of land. The Owner is The Mission Companies, P.O. Box 639, Addison, Texas 75001, contact: George Shirley (214) 906-0720. The property to be disturbed and the surrounding areas are currently paving and unimproved areas with grass. The land use is commercial. The main pollutant from the site during construction is erosion and sediment.

- The construction activities shall consist of excavation, utility line placement, storm sewer line and inlet construction, site grading, subgrade preparation and paving.

- A Notice of Intent shall be completed, submitted and posted at the project in plain view.

- After grading is completed, the contractor shall hydromulch all disturbed areas, unless specified differently in the landscape plans, and establish at least 90% grass coverage including necessary watering. The specific plant material proposed to protect fill and excavated slopes shall be as indicated on the landscape plans. Plant material must be suitable for use under local climate and soil conditions. In general, hydroseeding or sodding Bermuda grass is acceptable during the summer months (May 1 to August 30). Winter rye or fescue grass may be planted during times other than the summer months as temporary measure until such time as the permanent planting can be made.

- Prior to commencing any construction, a construction entrance and perimeter silt fence shall be installed at the location shown.

- All existing inlets shall be protected prior to start of construction. As inlets are completed, temporary sediment barriers shall be installed.

- Silt fence and inlet sediment barriers shall remain in place until vegetation has been completed.

- Disturbed areas that are seeded or sodded shall be checked periodically to see that grass coverage is properly maintained. Disturbed areas shall be watered, fertilized, and re-seeded or re-sodded, if necessary.

- Erosion control measures may only be placed in front of inlets, or in channels, drainageways or borrow ditches at risk of contractor. Contractor shall remain liable for any damage caused by the measures, including flooding damage which may occur due to blocked drainage. At the conclusion of any project, all channels, driveways and borrow ditches in the work zone shall be dredged of any sediment generated by the project of deposited as a result of erosion control measures.

- See separate soils report prepared by Reed Engineering Group for soil types and log of soil borings.