STORM SEWER CONSTRUCTION NOTES:

- All materials, construction, testing and workmanship shall conform to the Town of Addison Specifications and the Standard Specifications for Public Works Construction for North Central Texas, latest edition, except as noted herein and approved by the City.

- All public storm sewer pipe shall be a minimum of 18" and shall be Class III or Class IV RCP based upon depth of cover.  Class III RCP shall be used where the depth of cover, from ground line to top of pipe, is less than or equal to 10 feet.  Class IV RCP shall be used where the depth of cover is greater than 10 feet.  All storm sewer pipe shall be laid on a minimum of 4" of Grade No. 4 crushed stone (1" maximum diameter). The initial backfill shall consist of the same crushed stone to a minimum of the spring line of the pipe.  The remainder backfill must be clean and free of rocks & lumps of earth larger than 4" and of vegetation.

- The joints shall be constructed and jointed together in a manner that eliminates spill through of backfill.  Approved joint materials are concrete collars; plastic asphalt joint compound (cold applied); rubber gaskets; and preformed plastic gaskets (cold applied).

- The tops of all storm drainage inlets, manholes and junction structures shall have a round manhole cover with locking device.

- All drainage structures shall have a minimum compressive strength of 3600 psi at 28 days.

- All precast box culverts, drainage structures and RCP will require a certification from the manufacturer that the product meets the design requirements and 28 day compressive strength.

- All storm sewer systems with radii less than 100' shall use 4' long joints with beveled ends (radius pipe).

- See Town of Addison Standard Construction Details for all General Notes.